LITERATURE REVIEW

1. Overview of Existing Solutions

The domain of fuel theft detection has evolved significantly from mechanical locks to digital IoT solutions. Early research focused primarily on connectivity and basic alerting. Madhusudhana Rao et al. (2024) proposed a "Fuel Theft Detection System with Automated Calling and Alert Using GSM". Their work demonstrated the effectiveness of GSM modules for immediate remote notifications. However, the system was limited to SMS/Call alerts and lacked a real-time visual dashboard for trend analysis.

2. Evolution of IoT Monitoring

Subsequent studies shifted focus toward continuous monitoring. Chaudhari et al. (2024) and Deekshitha et al. (2024) developed "Smart Fuel Level Monitoring" systems using IoT devices. These projects successfully utilized ultrasonic and float sensors to digitize fuel levels, transmitting data to cloud platforms. While effective for basic logging, these systems relied heavily on static threshold values. Sharma et al. (2025) further optimized this approach with better cloud integration. However, a critical limitation across these studies is the high rate of false positives; simple threshold logic often misinterprets fuel "sloshing" (caused by vehicle vibrations or road bumps) as theft, leading to alert fatigue for operators.

3. Vehicle Security Integration

In a related domain, Pillutla et al. (2025) explored "Vehicle Theft Prevention by Operating Fuel Injector". While this addresses the immobilization of the vehicle during theft, it does not specifically resolve the issue of fuel pilferage from the tank itself during transit or authorized stops.

4. Critical Appraisal & Research Gap

A critical analysis of the existing literature reveals two major gaps:

- Lack of Intelligent Noise Filtering: Most existing systems use linear logic (e.g., "If level drops > X, alarm"). They lack the Machine Learning capabilities to distinguish between momentary sensor noise (vibration) and sustained fuel loss (theft).

- Privacy & Bandwidth Concerns: Standard IoT solutions transmit continuous streams of raw data to central servers. This centralized architecture consumes significant bandwidth and exposes sensitive operational data to potential privacy breaches.

5. Proposed Novelty

This project addresses these limitations by introducing Federated Machine Learning. Unlike previous works, this system employs an Isolation Forest algorithm to detect anomalies based on learned patterns rather than static rules. Furthermore, by simulating a federated architecture, it ensures that data processing occurs at the edge, offering a more secure, privacy-preserving, and accurate solution than those currently found in the literature.

REPORT ON THE PRESENT INVESTIGATION

1. Experimental Setup The experimental investigation was conducted using a custom-built Internet of Things (IoT) testbed designed to simulate real-world fuel tank dynamics. The hardware setup consists of a NodeMCU (ESP8266) Wi-Fi microcontroller acting as the primary edge processing unit. A high-precision HC-SR04 Ultrasonic Sensor is interfaced with the NodeMCU (Trig on GPIO14, Echo on GPIO12) to measure the fuel level distance with a resolution of 0.3 cm. For immediate physical feedback, a piezoelectric buzzer and a visual status LED are connected to the microcontroller’s digital output pins.

The system is powered via a 5V USB interface, simulating the power supply available in a vehicle's dashboard. The "Central Server" is hosted on a standard laptop running a Python Flask backend, which acts as the aggregator for the Federated Learning simulation. The connectivity is established over a local 2.4 GHz WiFi network, mimicking the wireless data transmission that would occur via GSM/4G in a deployed commercial environment. A scaled model fuel tank (height: 11 cm) was utilized for controlled testing of fuel levels, sloshing effects, and drainage scenarios.

2. Methodologies Developed and Adopted The core methodology adopted for this investigation is a Hybrid Edge-Cloud Architecture combined with a Federated Learning (FL) Simulation. Unlike traditional systems that rely solely on cloud processing, this project adopts a distributed approach:

- Edge Data Acquisition: The NodeMCU functions as a "Thin Client," performing initial signal filtering (rejecting 0cm/timeout errors) before packaging validated data into JSON payloads.

- Privacy-Preserving Federated Simulation: A novel methodology was adopted to address data privacy. Instead of assuming raw data storage, the system architecture simulates a Federated Learning environment. The backend processes incoming data streams to compute local statistical variances, which are then "aggregated" by a central Federated Server module using the FedAvg algorithm. This demonstrates how a fleet of trucks could update a global theft detection model without exposing sensitive route or consumption data.

- Unsupervised Anomaly Detection: The investigation moved beyond static thresholding by adopting the Isolation Forest algorithm. This unsupervised learning technique was selected for its ability to isolate anomalies (theft) based on deviation from the majority data distribution (normal usage), allowing the system to adapt to varying "normal" fuel levels without manual recalibration.

3. Procedures Adopted The investigation followed a systematic procedure to validate the system's reliability:

- Baseline Calibration: Upon startup, the system initiates a "locking" procedure where it averages the first 5 seconds of sensor data to establish a "Stable Baseline" (Anchor Point).

- Continuous Polling: The server polls the edge node data every 0.5 seconds. Incoming data is passed through two parallel analysis pipelines: a deterministic logic check (Ground Truth) and the Isolation Forest prediction model.

- Signal Debouncing: To differentiate between vehicle vibrations (shaking) and actual theft, a strict Confirmation Window procedure was adopted. An alert is only triggered if an anomaly persists for three consecutive reading cycles.

- Heartbeat Monitoring: A safety procedure was implemented where the server tracks the time since the last data packet. If communication is lost for more than 5 seconds, the system automatically transitions to an "OFFLINE" state, preventing false "Normal" readings during power failures.

4. Techniques Developed Several specific techniques were developed during the investigation to overcome hardware limitations and environmental noise:

- Sticky Baseline Logic (The "Anchor" Technique): To detect slow-draining leaks that might evade standard detection, a "Sticky Baseline" technique was developed. The system remembers the last known stable fuel level and compares current readings against this anchor rather than the immediately preceding value. This ensures that gradual fuel theft (e.g., siphoning via a small tube) is detected as the cumulative difference grows, triggering an alarm once the aggregate loss exceeds the 2cm threshold.

- Hybrid Verification System: A dual-verification technique was engineered to maximize accuracy. The system calculates an "Accuracy Metric" in real-time by cross-referencing the Machine Learning model's unsupervised prediction against the deterministic logic rules. This technique provides a live confidence score (displayed on the dashboard), offering operators transparency regarding the model's performance.

- Automated Actuation Timer: To prevent alarm fatigue, an automated latching technique was coded into the alert logic. Upon confirming theft, the system triggers the hardware buzzer for a fixed duration of 5 seconds and then automatically silences it, continuing to log the theft status silently on the dashboard. This ensures the driver is alerted without creating a continuous distraction during the emergency response.

WORK TO BE CARRIED OUT FOR THE NEXT ONE MONTH

Week 1: Developing and Tuning the ML Model

- Implement the Federated Averaging (FedAvg) logic within the Python backend to simulate the aggregation of local model weights.

- Train the Isolation Forest model with the newly collected dataset to define precise "Normal" vs. "Theft" boundaries.

- Optimize the model's contamination parameter to reduce false positives based on the specific dimensions of the fuel tank (11 cm).

Week 2: Connecting Hardware & Embedded Programming

- Finalize the circuit connections by transitioning from a prototyping breadboard to a permanent Soldered Perfboard setup.

- Refine the NodeMCU (Arduino C++) firmware to include the "Debouncing Algorithm" directly in the embedded code, filtering out sensor vibrations.

- Ensure reliable power distribution to the Ultrasonic Sensor, Buzzer, and LED to prevent voltage drops and sensor timeouts.

Week 3: Web Interface Development

- Develop a responsive Real-Time Dashboard using React.js to visualize the live data stream from the hardware.

- Integrate the Recharts library to render a dynamic line chart that displays fuel level trends and anomalies instantly.

- Create visual status indicators (Green/Red/Grey Cards) that update automatically based on the API response from the Python server.

Week 4: System Integration and Testing

- Conduct Latency Testing to ensure the web interface updates within 1 second of a hardware event.

- Execute Scenario-Based Testing (e.g., Simulated Shake vs. Actual Drain) to validate the system's overall reliability and accuracy.

Visible Research Output During the Review Period